Harini G

192524011

LOGICAL, BIT-MANIPULATION AND SHIFT OPERATIONS
USING 8085 MICROPROCESSOR

a. PROGRAM A – 8-BIT LOGICAL AND MASKING OPERATIONS

Aim: To write and execute an 8085 assembly language program to perform AND, OR, XOR, complement and masking operations on 8-bit data.

Apparatus Required:

- 8085 microprocessor simulator

- Computer system

Input:

- X = A7H stored at memory location 4500H

- Y = 3CH stored at memory location 4501H

Algorithm:

- Load the first 8-bit data into the accumulator.

- Perform AND, OR and XOR operations with the second data.

- Perform the complement operation on the first data.

- Apply a mask to extract the lower nibble.

- Store each result in the specified memory location.

- Stop the program.

Program:

LDA 4500H

MOV B A

LDA 4501H

ANA B

STA 4502H

LDA 4500H

MOV B A

LDA 4501H

ORA B

STA 4503H

LDA 4500H

MOV B A

LDA 4501H

XRA B

STA 4504H

LDA 4500H

CMA

STA 4505H

LDA 4500H

ANI 0FH

STA 4506H

HLT

Input and Output:

| Memory location | Data |
| --- | --- |
| 4500H | A7H |
| 4501H | 3CH |
| 4502H – AND result | 24H |
| 4503H – OR result | BFH |
| 4504H – XOR result | 9BH |
| 4505H – Complement | 58H |
| 4506H – Lower nibble | 07H |

Result: Thus, the 8-bit logical and masking operations were performed successfully using the 8085 microprocessor.

Input: X = A7H at 4500H, Y = 3CH at 4501H.

8085 simulator-style screenshot illustration

b. PROGRAM A – 16-BIT ONE’S AND TWO’S COMPLEMENT

Aim: To write and execute an 8085 assembly language program to find the one’s complement and two’s complement of a 16-bit number.

Apparatus Required:

- 8085 microprocessor simulator

- Computer system

Input:

- 16-bit number W = 1234H

- Lower byte = 34H at 450AH

- Higher byte = 12H at 450BH

Algorithm:

- Load the lower byte of the 16-bit number.

- Complement the lower byte and store the result.

- Complement the higher byte and store the result.

- Add 1 to the complemented lower byte.

- If a carry occurs, increment the complemented higher byte.

- Store the two’s complement result and stop the program.

Program:

LDA 450AH

CMA

STA 450CH

LDA 450BH

CMA

STA 450DH

LDA 450AH

CMA

ADI 01H

STA 450EH

JNC SKIP

LDA 450BH

CMA

INR A

STA 450FH

JMP STOP

SKIP: LDA 450BH

CMA

STA 450FH

STOP: HLT

Input and Output:

| Memory location | Data |
| --- | --- |
| 450AH | 34H |
| 450BH | 12H |
| 450CH – One’s complement (low) | CBH |
| 450DH – One’s complement (high) | EDH |
| 450EH – Two’s complement (low) | CCH |
| 450FH – Two’s complement (high) | EDH |

Result: Thus, the one’s complement and two’s complement of the given 16-bit number were obtained successfully.

Input: W = 1234H (34H at 450AH, 12H at 450BH).

8085 simulator-style screenshot illustration

c. PROGRAM B – 8-BIT BIT SETTING, CLEARING, ROTATE AND SHIFT

Aim: To write and execute an 8085 assembly language program to perform bit setting, bit clearing, rotation and shift operations on an 8-bit number.

Apparatus Required:

- 8085 microprocessor simulator

- Computer system

Input:

- Data D = CAH stored at memory location 4510H

Algorithm:

- Load the given 8-bit data into the accumulator.

- Set bit 0 using the OR operation.

- Clear bit 3 using the AND operation with a mask.

- Perform left and right rotate operations.

- Perform left and right shift operations.

- Store the results in memory and stop the program.

Program:

LDA 4510H

ORI 01H

STA 4511H

LDA 4510H

ANI F7H

STA 4512H

LDA 4510H

RLC

STA 4513H

LDA 4510H

RRC

STA 4514H

LDA 4510H

ADD A

STA 4515H

LDA 4510H

STC

CMC

RAR

STA 4516H

HLT

Input and Output:

| Memory location | Operation | Result |
| --- | --- | --- |
| 4510H | Input data | CAH |
| 4511H | Set bit 0 | CBH |
| 4512H | Clear bit 3 | C2H |
| 4513H | Rotate left | 95H |
| 4514H | Rotate right | 65H |
| 4515H | Shift left | 94H |
| 4516H | Shift right | 65H |

Result: Thus, bit setting, bit clearing, rotation and shift operations were performed successfully using the 8085 microprocessor.

Input: D = CAH at 4510H.

8085 simulator-style screenshot illustration

d. PROGRAM B – 16-BIT LEFT SHIFT

Aim: To write and execute an 8085 assembly language program to perform a 16-bit left shift operation.

Apparatus Required:

- 8085 microprocessor simulator

- Computer system

Input:

- 16-bit number W = 9234H

- Lower byte = 34H at 4520H

- Higher byte = 92H at 4521H

Algorithm:

- Load the 16-bit number into the HL register pair.

- Add the HL register pair to itself using the DAD H instruction.

- The operation shifts the 16-bit value one bit to the left.

- Store the result in the specified memory locations.

- Stop the program.

Program:

LHLD 4520H

DAD H

SHLD 4522H

HLT

Input and Output:

| Memory location | Data |
| --- | --- |
| 4520H | 34H |
| 4521H | 92H |
| 4522H – Result (low byte) | 68H |
| 4523H – Result (high byte) | 24H |
| Carry flag | 1 |

Result: Thus, the 16-bit left shift operation was performed successfully using the 8085 microprocessor.

Input: W = 9234H (34H at 4520H, 92H at 4521H).

8085 simulator-style screenshot illustration